Devin Fromond
CS 3220  |  A 
2/19/2026

Lab 2 Report: Performance Measurement and Optimization

The motivation for Lab 2 Part Two is to improve the branch-prediction accuracy of the TinyRV pipeline on the towers.mem benchmark. In the report below, we will document three design variants and discuss their impact(s).

We observe three predictor parameters (located in define.vh) which influence the table below:

- BHRBITS (global history width) which sets the PHT depth (PHT_ENTRIES = 2^BHRBITS)

- BTB_ENTRIES / BTB_INDEX_BITS for BTB capacity

- USE_PHT_PC_HASH flag that XORs two PC slices before combining with global history

| Label | BHRBITS | BTB_ENTRIES | USE_PHT_PC_HASH | Notes | Accuracy |
| --- | --- | --- | --- | --- | --- |
| Baseline | 8 | 16 | FALSE | Matches the reference FE-stage predictor | 37.03% |
| Variant A (Larger BTB) | 8 | 32 | FALSE | Doubled BTB capacity to cut down direct-mapped conflicts | 46.42% |
| Variant B (Wider GShare) | 10 | 32 | FALSE | Increased global history depth | 46.22% |
| Variant C (Best Design) | 10 | 64 | TRUE | Adds both wider history and hashed PC slices with a larger BTB | 58.22% |

Baseline: The starter predictor passes the 30% accuracy threshold but observes significant BTB thrashing since only 16 entries are available for the ~990 branch/jumps found in towers.mem.

Variant A: Increasing the BTB to 32 entries boosts accuracy by ~9% without touching PHT. This confirms the previous assumption that capacity/conflict misses were the primary issue in Baseline.

Variant B: Increasing BHRBITS to 10 – allowing for a 1024-entry PHT – without increasing the BTB does not provide additional benefit beyond Variant A. The predictor is history-rich but still runs into issues when multiple status branches alias to the same BTB slot.

Variant C: When combining a 64-entry BTB with hashed PC slices, BTB conflicts and PHT aliasing are almost entirely eliminated. We observe the accuracy increases to ~58%, resulting in the best design of the three variants.